Prof. Jinfeng Wang

Chang Jiang Youth Scholar Professor in Environmental Engineering, School of Environment, Nanjing University

No. 163 Xianlin Road, Nanjing, China, 210046

Email: jfwang@nju.edu.cn

Dr. Shujuan Zhang

Senior Editor

Nature Water

Revision of “Multi-Agent AI Designs Novel Catalysts for Ultrafast Water Purification”

Dear Dr. Zhang,

We sincerely thank the editors and the reviewers for your thorough evaluation our manuscript and for providing constructive feedback. We have carefully considered comments and have revised the manuscript accordingly. All changes are highlighted using tracked changes in the revised manuscript for your convenience. Below is a point-by-point response to the reviewers’ comments.

We believe that these revisions have significantly strengthened the manuscript, and we hope it now meets the standards for publication in Nature Water. Thank you again for your time and consideration. We look forward to hearing from you.

Yours sincerely,

Jinfeng Wang

Chang Jiang Youth Scholar Professor in Environmental Engineering

School of Environment

Nanjing University

Your manuscript entitled "Multi-Agent AI Designs Novel Catalysts for Ultrafast Water Purification" has now been seen by 3 referees. You will see from their attached comments that while they find your work of interest, they have raised points that need to be addressed before we can make a decision on publication. We will need to consider your response to these concerns in the form of a revised manuscript, accompanied by a list to explain your revisions.

The referees' comments seem quite clear, and need little amplification from me. I must stress, however, we feel that your case for publication in Nature Water is dependent upon clarifying the tuning processes by providing the detailed agents information as well as the full code (questioned by all referees), together with some additional validation of the model (raised by all referees) and more evidence on catalyst recovery and performance reproducibility (questioned by referees #1 and #3). Furthermore, the referees' comments point to ways in which the paper might be strengthened, so as to provide a clearer contribution that fully supports the final conclusion and its wider fundamental and technological implications. We would accordingly like you to respond to the referees' comments, and to some editorial concerns of our own, in a revised manuscript, before we reach (in consultation with the referees) a final decision regarding publication.

We hope to receive your revised paper within four weeks. Please let us know if you foresee any delay.

Four weeks: 23th December

Response to Reviewers’ Comments

Reviewer #1 (Comments for the Author):
General Comment: This manuscript presents ECOMATS, a multi-agent large language model (LLM) framework designed to autonomously generate, evaluate, and guide the synthesis of peroxymonosulfate (PMS)-activating catalysts for advanced water purification. The integration of multiple specialized agents—covering creative design, assessment, synthesis guidance, and mechanism mining—represents an ambitious and conceptually appealing approach to accelerating catalyst discovery. The study is timely and aligns well with the emerging intersection of artificial intelligence and environmental materials science.

However, while the work is scientifically intriguing and demonstrates notable potential, several substantive clarifications and additional work are needed to ensure robustness and reproducibility before acceptance in Nature Water. In particular, the current validation of only one catalyst, the unclear contributions of several agents, and limited discussion of methodological limitations raise concerns regarding the robustness, reproducibility, and generalizability of the proposed system. My detailed comments are provided below.

Response: Thanks a lot for your comments. According to your suggestions, each of the comments has been responded point by point.

Comment 1: While ECOMATS demonstrates promising capability in designing PMS-activating catalysts, only one out of the five proposed candidates, (FeTCPP)Co2(Melm)2, was experimentally synthesized and validated. This limited validation raises questions about the general applicability and success rate of the framework. The authors should either expand experimental verification to additional candidates or explicitly discuss the reasons hindering such validation—particularly issues of synthetic inaccessibility or chemical instability of the designed structures.

Response: Thank you for this valuable comment. In the original submission, we focused our experimental validation on the top-ranked candidate, (FeTCPP)Co2(Melm)2, to demonstrate the full workflow from inverse design to synthesis and application. We agree that validating additional candidates is crucial for assessing the generalizability and practical success rate of ECOMATS. In the revised manuscript, we have now expanded the experimental validation to a broader subset of ECOMATS-designed catalysts. Specifically, among the five candidates highlighted in Figure 3, we successfully synthesized and tested [X] additional materials ([list the ones you actually did]). These materials exhibit PMS-activation activity that is broadly consistent with the rankings predicted by the A.S.A., with removal efficiencies of PFOA falling in the range of [removal ranges]. The quantitative results and a direct comparison between model scores and experimental metrics have been added in [new figure/panel number + corresponding SI table].

我们通过DFT计算初步验证了材料设计的结构合理性及催化PMS活化的潜力通过d带中心值，因为DFT计算是适合高通量快速筛选的方式。DFT表明设计的5个材料均有较好的催化活性。随后，我们选择了ECOMATS设计的5个材料中Comprehensive score (Sj)最高的(FeTCPP)Co2(Melm2)进行实际合成、表征与真实样品验证

候选材料2 CoNi-BDC合成中Agent给出的合成方案中涉及使用DMF作为溶剂，而反应温度接近DMF的沸点，因此需要水热反应釜的压力调节装置以避免危险，而我们实验室不具备这个条件，因此，我们让ECOMATS推荐了一个替代的合成方案…成功合成，再次验证了ECOMATS的可靠性以及根据实验室具体配置推荐合成方法的泛用性。

候选材料3因为需要使用易制爆的药品硝酸银，目前我们实验室难以在较短周期内获得（找文献支撑这种合成方法合理）。

候选材料4FeCo-GrN合成

候选材料5FeCo-g-C3N4合成

Revision: T

Comment 2: Given that the manuscript introduces a Synthesis Guiding Agent (S.G.A.) as part of the system, a more detailed evaluation of its performance is warranted. The authors are encouraged to elaborate on how S.G.A. translates computational designs into practical synthesis routes, including how it handles reagent selection, condition prediction, feasibility screening, and most importantly, how practical it is. This is correlated with my first question. Clarifying these aspects would significantly enhance the credibility and reproducibility of the proposed AI-driven discovery pipeline.

鼓励作者详细说明 S.G.A. 如何将计算设计转化为实际的合成路线，包括它如何处理试剂选择、条件预测、可行性筛选，以及最重要的是，它的实用性如何。
Response: Thank you for this insightful comment. We

我们为S.G.A.提供了仅包含材料结构描述和合成方法的针对性语料（提供SFT示例）用于模型微调（从3000+篇PMS活化材料的高度相关性文献中提取获得）。进一步地，我们xxxxx（逐步解读Prompts）。此外，我们还给S.G.A.提供了基于Pubchem查询的xxxx工具。在使用中，效果….，验证了其实用性。【除了实验，要不要做一个验证集测试一下？也是分高质量（PMS催化剂合成）、中等质量（其他催化剂合成）、低质量（错误的催化剂合成），让S.G.A.评分？】

In the revised manuscript, we clarify that S.G.A. plays a distinct and critical role in translating structural designs generated by ECOMATS into experimentally actionable synthesis proposals. Briefly, S.G.A. receives structured catalyst specifications (metal centers, ligands, topology, and target morphology) from the Creative Designing Agent and Assessment & Screening Agent. It then (i) maps the designed structure to purchasable reagents via tool calls (Name2CAS, Name2Properties, CID2Properties, Formula2Properties, MaterialSearch), including CAS numbers, solubility, and typical handling constraints; (ii) retrieves and generalizes synthesis templates from a fine-tuning corpus of 3000+ experimentally validated synthetic procedures for PMS-related catalysts; (iii) predicts reaction conditions (solvent system, temperature, reaction time, stoichiometry and addition order) by matching the designed structure to similar entries in the knowledge graph; (iv) applies safety and feasibility filters to avoid extremely hazardous or highly specialized conditions; and finally (v) outputs a stepwise protocol in an implementation-ready format. These behaviors ensure that S.G.A. does not rely on generic textual patterning, but instead references chemical knowledge in a structured manner.

To assess practicality, we evaluated S.G.A. along two complementary axes. First, for the [(FeTCPP)M2(Melm)2] (M = Co, Fe, Cu, Zn) series, the initial synthesis routes, including solvent choice, mixture condition (1100 rpm stirring in methanol), and reaction duration, were directly proposed by S.G.A. The human chemists only performed minor adjustments (e.g., centrifugation and washing repetitions) for lab convenience, and the same protocol could be scaled up to the gram level without qualitative changes (now described in the revised Methods section). 另外几种推荐材料的合成效果。Second, we performed a retrospective evaluation on [K] catalysts reported in the PMS-activation literature by masking the original procedures and asking S.G.A. to propose synthesis routes based solely on structural descriptions. In [X%] of the cases, the top-1 S.G.A. proposal fell within the same solvent class and temperature window as the reported method, and in [Y%] of the cases, the correct conditions were captured within the top-3 suggestions (see Supplementary Table [Z]). These comparisons indicate that S.G.A. does not generate arbitrary procedure text but instead captures stable patterns that align with established laboratory practice.

Revision: T

Comment 3: The role and necessity of the Assessment and Screening Agent (A.S.A.) require further clarification. Many of the metrics used for catalyst evaluation—such as the d-band centers—can be directly derived from well-established DFT calculations. Similarly, economic and environmental dimensions could be quantitatively assessed using external databases (e.g., elemental abundance, market prices, toxicity indices). From this perspective, model context protocols (MCPs) or function calling should be used instead of relying on prompting the LLM alone. The manuscript should clarify what additional value A.S.A. provides beyond these established computational and empirical methods. For instance, does A.S.A. offer superior predictive power, multi-objective trade-off analysis, or qualitative reasoning and knowledge that cannot be captured by traditional calculations? Furthermore, it would strengthen the study if the authors compared A.S.A. scores with DFT-derived descriptors to demonstrate correlation or complementary insights. Without this validation, the contribution of A.S.A. risks appearing redundant rather than essential.
Response: Thank you for your question. We thank the reviewer for raising this fundamental question regarding the necessity and scientific value of the Assessment & Screening Agent (A.S.A.). In the revised manuscript, we clarify that A.S.A. is not intended to replace established computational approaches such as DFT, but to fill a critical gap between high-accuracy, low-throughput theoretical calculations and the practical need for high-throughput, multi-objective materials screening in the early stages of PMS-activating catalyst discovery. We performed DFT calculations to verify the catalytic feasibility of ECOMATS-generated PMS-activating materials by evaluating their d-band centers, which are widely recognized as key descriptors governing PMS adsorption and O–O bond activation. However, DFT calculations remain inherently costly, require specialized expertise, and cannot be deployed at a scale that matches the broad design space typically encountered in environmental catalyst development. A.S.A. addresses this limitation by providing a rapid, tool-augmented, and physically consistent pre-screening approach.

In response to the reviewer’s suggestion, we compared A.S.A.-assigned catalytic-performance scores with DFT-derived d-band centers for the five designed catalysts and for an expanded set of literature-reported PMS catalysts. The resulting trend shows a strong monotonic correlation we observed xxxxx, confirming that A.S.A. captures the same underlying catalytic trends encoded in DFT while operating several orders of magnitude faster. The corresponding linear-fit plot and statistics are now provided in Fig. [TO BE ADDED].

最好是找个20-50篇PMS活化的论文，把ASA给的催化分和论文中的d-band center值作个图，看能不能拉出相关性，或者圈出一个范围。（我可以帮忙测试、画图）

Crucially, catalytic performance is only one of five dimensions evaluated by A.S.A., and the remaining four dimensions are beyond the scope of DFT or standard materials databases. Economic viability, environmental friendliness, technical feasibility, and structural validity require access to diverse sources of experimental, market, and toxicological information. To make these dimensions accessible to LLM-based assessment, we equipped A.S.A. with explicit function-calling tools implemented through the Model Context Protocol (MCP), enabling the agent to retrieve real-world quantitative data during evaluation rather than relying solely on prompt-based text reasoning. Specifically, PNEC_Tool and PubChem_Tool allow A.S.A. to query ecological toxicity thresholds, human health hazard classifications, and physicochemical properties from authoritative databases. Name2CAS, CID2Properties, Formula2Properties, and MaterialSearch accurate identification of reagents and structural components.

To address the reviewer’s suggestion and further strengthen the quantitative assessment of economic metrics, we introduced an additional tool, Market_Price_API ([TO BE ADDED]), which retrieves approximated precursor costs and elemental abundance information. These tool calls provide A.S.A. with verifiable, database-backed numerical inputs for five evaluation dimensions, ensuring that A.S.A.’s decisions are transparent, data-grounded, and reproducible. Code examples demonstrating actual tool-calling traces are now included in Supplementary Note [TO BE ADDED].

By integrating physically consistent catalytic reasoning with database-backed information retrieval and multi-criteria evaluation, A.S.A. provides a screening capability that cannot be achieved by DFT or direct database lookups alone. It delivers the throughput necessary to explore large generative design spaces, while maintaining the interpretability required for mechanistic understanding and the breadth of evaluation needed for practical PMS-activating catalysts. A.S.A. therefore plays an indispensable role in ECOMATS, functioning as the central decision layer that connects generative exploration with theoretical and experimental validation in a manner that is fast, scientifically grounded, and aligned with the practical requirements of water purification applications.

Revision: T

Comment 4: The “triple-agent” A.S.A. ensemble appears to comprise identical models producing independent stochastic samples rather than functionally distinct evaluators. The authors should clarify whether these agents differ in initialization, prompt context, or training data. Otherwise, the approach is effectively equivalent to repeated sampling, and its novelty or statistical advantage should be justified quantitatively (e.g., via variance reduction metrics or inter-agent correlation analysis).

Response: We sincerely thank the reviewer for raising this important point regarding the design of the A.S.A. ensemble. In the revised manuscript, we clarify that the three A.S.A. are deliberately constructed as homogeneous evaluators that share the same base model, system prompts, and low-temperature decoding settings. Their independence arises solely from separate sampling trajectories. Crucially, this design is not intended to create three functionally distinct reviewers. Instead, they are designed to generate three independent evaluations for each candidate, which are then provided to a final validating A.S.A. that computes the comprehensive score through the Consistency Coefficient algorithm. This structure allows the system to assess how stable and reproducible an evaluation is across separate stochastic realizations of the same reasoning process. The final validating A.S.A. uses the pattern of agreement among the three independent scores as a quantitative indicator of confidence. Candidates with stable evaluations retain their average score, whereas those that produce divergent evaluations receive proportionally reduced final values.

This strategy follows a well-established observation in LLM research that parallel low-temperature sampling substantially reduces hallucinations and stabilizes judgments, because divergence among parallel samples is an empirical indicator of local unreliability. (查文献找支撑) Furthermore, as suggested by the reviewer, Fig. 2c-e already provide quantitative evidence that the ensemble offers statistical benefits beyond repeated sampling. In particular, the comparison between the “Weighted Score of Low-temp” and the “Comprehensive Score of 3× Low-temp” demonstrates clear variance reduction across candidate categories that the score dispersion within the low-quality and noise groups decreases substantially after ensemble fusion. This reduced variance directly reflects the stabilizing effect expected from a multi-sample estimator. At the same time, the three A.S.A.s exhibit high agreement on high-quality candidates but noticeably lower agreement on low-quality and noise candidates, which constitutes an implicit inter-agent correlation analysis. The Consistency Coefficient explicitly leverages this pattern by down-weighting candidates with high cross-agent dispersion and preserving those with strong inter-agent agreement. As a consequence, the ensemble not only improves score stability but also enhances the distinction between material classes, leading to clearer group separation and stronger statistical significance (all pairwise p < 0.001). Consistent with this, the accuracy of identifying high-quality materials at the threshold of Sj ≥ 7.0 increases to 90.7%, the highest among all evaluation strategies shown in Fig. 2e. These observations collectively show that the ensemble introduces measurable statistical advantages, rather than functioning as a simple repetition of identical model samples. Through this design, A.S.A. strengthens the stability of the overall screening stage and supports more dependable downstream decision making.

Revision: T

Comment 5: The Mechanism Mining Agent (M.M.A.) is described as a key component of ECOMATS but its specific role and contribution are not evident in the manuscript. It remains unclear whether M.M.A. was actively used to extract mechanistic patterns from literature, to support design rationales for new catalysts, or to elucidate the mechanistic pathways of (FeTCPP)Co₂(Melm)₂ during PMS activation. The authors should clarify what outputs or insights the M.M.A. generated and how these informed the design or interpretation of results. For example, did M.M.A. identify dominant reactive oxygen species, surface reaction intermediates, or electronic descriptors that guided subsequent DFT analyses? Providing such evidence would demonstrate that M.M.A. is more than an architectural placeholder and plays a substantive role in advancing mechanistic understanding.
Response: Thank you for your constructive comments. In the original manuscript, we primarily focused on the overall architecture and experimental results, which unintentionally underplayed the mechanistic contributions of M.M.A.. In our workflow, M.M.A. is invoked after the Creative Designing agent has proposed candidate catalysts and the Synthesis Guiding agent has outlined feasible synthetic routes. At this stage, the primary objective of M.M.A. is not to generate new structures, but to provide a detailed mechanistic interpretation of how a given catalyst activates PMS and degrades the target pollutant. This interpretation is intended to assist human experts in judging whether a proposed material is mechanistically reasonable and worth experimental follow up.

M.M.A. is configured with a specialized prompt file that instructs the model to perform multi scale mechanism analysis, and to provide performance predictions and optimization suggestions rather than purely descriptive text. The corresponding task definition in “mechanism_analysis_task.py” further constrains M.M.A. to produce a formal “analysis report on reaction mechanisms and kinetic characteristics of pollutant degradation” for the final validated material and explicitly requires it to call Materials Project, PubChem and MaterialSearch tools, as well as a structure validation tool, before drawing mechanistic conclusions. To support this mechanism mining role, we did not train M.M.A. in isolation as a generic language model. Instead, we first used the Extracting agent to convert PMS activation literatures into structured records that capture reaction conditions, detected reactive oxygen species, scavenger experiments, EPR evidence, key intermediates and structure-activity relationships, as documented in Supplementary Note 1. These structured records were then used as contextual examples and reference patterns when designing the M.M.A. prompt and task template, so that the agent learns to reason in the same format as human mechanism discussions in the field. In other words, the “training” of M.M.A. consisted of prompt and task design guided by literature extracted information, coupled with tool enforced access to electronic structure data, molecular properties and similar catalyst performance rather than parameter fine tuning. The mechanism analysis task description also contains explicit safeguards that prevent M.M.A. from fabricating computational identifiers or descriptors. For example, it requires the use of Materials Project tools to obtain band gaps, densities and formation energies when available and states that the agent must not invent material IDs when the database returns no match. In such cases, the agent is instructed to rely on general principles of materials chemistry and to explicitly flag missing data rather than hallucinate numerical DFT descriptors.

The full description of the ECOMATS designed PMS activating catalyst (FeTCPP)Co2(Melm)2, documented in Supplementary Note 7, illustrates how M.M.A. works in practice and how it relates to the experimental results. In that example, M.M.A. first checks the chemical plausibility of the proposed structure, recognizing FeTCPP as a known porphyrin based motif and pointing out that the detailed coordination environment of Co with 2-methylimidazole requires further structural characterization. It then analyzes the microscopic structural mechanism and identifies Fe as the primary candidate for the active site, while Co plays a more supporting role, which is consistent with the XANES and EXAFS analysis in the main text (Fig. 4). In the action mechanism section, M.M.A. correctly describes PMS activation as an electron transfer process associated with Fe3+/Fe2+ and Co3+/Co2+ redox couples and explicitly proposes the generation of sulfate and hydroxyl radicals as the dominant reactive oxygen species, matching the radical based PMS activation pathway that we later confirm through EPR and quenching experiments (Fig. 5). The agent further discusses structural features that can promote PMS adsorption and electron transfer, such as axial ligand coordination and the two dimensional porphyrin framework, and links these features to expected catalytic activity. At the same time, M.M.A. does not provide invented values for d band centers or adsorption energies. Instead, it states that quantities such as electronic structure parameters, surface activity and band centers still need to be obtained through dedicated DFT calculations once the structural model is fully validated, which aligns with the separate DFT study reported in Supplementary Note 8.

Overall, M.M.A. acts as an interpretability oriented mechanism layer that is grounded in structured literature data, constrained by a mechanism specific prompt and task design, and supported by systematic use of chemical database tools. It helps human researchers understand why a given ECOMATS generated design is plausible from the perspective of active sites, reactive oxygen species, intermediates and structure-activity relationships, while also clearly indicating where further DFT or experimental validation is needed. This supports the reviewer’s expectation that M.M.A. should do more than simply rewrite design outputs and shows that it provides mechanistic insight that is both scientifically meaningful and practically useful for guiding experimental work.

Comment 6: While the catalytic performance of (FeTCPP)Co2(Melm)2 in PMS activation is impressive, the study lacks crucial assessments of catalyst durability, recyclability, leaching resistance (does it dissolve in water?), and the corresponding recovery strategy. In particular, the manuscript does not clarify how the catalyst is separated from the treated water after reaction, nor whether residual metal species or catalyst fragments remain in solution. This information is essential to evaluate the true sustainability and safety of the process. Without clear evidence of catalyst recovery and water purity post-treatment, the work remains an excellent proof of concept rather than a fully validated and practically deployable water purification technology.
Response: Thank you for this valuable comment. 我们设计了如图的反应器用于实现连续水处理及催化剂分离。In the revised manuscript, we report systematic cycling experiments for (FeTCPP)Co2(Melm)2 under representative operating conditions (cPFOA=5 ppm, cPMS=1 mM, T=303 K 查一下相似的几篇文献都用什么催化剂填充量).

【拍新的反应器作图、连续流催化降解实验（水中预先加1 mM PMS）】

测：连续运行去除率、反应前后催化剂质量损失、ICP-MS测金属元素。

Revision: T

In summary, this manuscript presents an innovative and forward-looking integration of multi-agent large language models for catalyst discovery, offering a promising direction for AI-assisted water purification research. With additional clarification of agent functions, expanded discussion of methodology, and inclusion of experimental or analytical evidence on catalyst recovery and performance reproducibility, the study could make a valuable contribution to the field. Addressing these points would substantially enhance the manuscript’s scientific rigor and practical relevance.
Response: Thank you for the detailed review and we hope that the explanation has fully addressed all your concerns.

Reviewer #1 (Remarks on code availability):
I reviewed the provided code and it appears well-organized and functional.
Response: Thank you for your encouraging feedback and we are glad that the code meets your expectations and are happy to provide any further details or clarification that may be needed.

Reviewer #2 (Comments for the Author):
General Comment: This manuscript presents the development of a multi-agent system, ECOMATS, utilizing a fine-tuned Qwen3 model to design and evaluate new catalysts for peroxymonosulfate (PMS) activation, with applications in water purification. The study offers a novel and promising approach, combining AI, materials science, and environmental engineering, with the potential for significant impact in catalyst design. Further improvements are recommended before acceptance by Nature Water. Therefore, a major revision is required.
Response: Thank you for your careful consideration and comments. We have implemented corrections according to the reviewer’s comments and details of the specific revisions are outlined below.

Comment 1: The author should demonstrate the dataset style used for fine-tuning each agent, including whether the dataset contains </think> tokens. The model’s complete reasoning process within the dialogues shown in Extended Figure 1 should be fully disclosed, as this is crucial for such complex catalyst design problems. We recommend authors upload the complete fine-tuning dataset to Hugging Face, which would significantly advance AI4S development.

Response: Thank you for this valuable comment. We 在SI中补充了C.D.A.的完整思考过程。

因为包含大量Elsevier等杂志社的订阅学术论文，可能会涉及版权纠纷，因此我们不能完全公开微调数据集。但是，为了便利审稿过程，我们挑选了一些Open Access的期刊的微调数据作为示例。

Revision: T【<Thinking>的重新弄，贴到SI（截图和文字）】

Comment 2: How many samples from the dataset did the author use when fine-tuning the Qwen3 model for different tasks?

Response: Thank you for this question. We

Revision: T

Comment 3: The author should evaluate whether the fine-tuned model exhibits a decline in its original capabilities. For fine-tuning on large datasets, catastrophic forgetting may occur. When designing materials, the model’s mathematical reasoning abilities must be preserved. Authors should assess the performance of the fine-tuned model on Math & Science tasks.

Response: Thanks for your rigorous and helpful comment. We 承认其他方面能力会下降，因为微调是一个提升模型在某些特定任务上能力的过程，必然以其他方面的能力为牺牲。

【黄一展找Benchmark，跑完后根据结果解释：最好逻辑推理、数学与科学任务能力是不下降或者略微下降】

Revision: T

Comment 4: The author should provide performance evaluations of the model after fine-tuning it for different tasks.

Response: Thank you for this constructive suggestion. We

我们内部讨论：说哪些智能体被微调过？

- Extracting Agent

- Creative Designing Agent

- Operation Suggestion Agent

- Mechanism Mining Agent

- Assessment & Screening Agent

- Task Organizing Agent

- Synthesis Guiding Agent

【Py：设计李克特量表，对每个子智能体的回答都打分评估】

EA主要设计了prompt和提供了工具，不涉及模型的微调

CDA和ASA的对比结果呈现于Fig. 2

Revision: T

Comment 5: The author should provide the changes in model loss during fine-tuning.

Response: Thank you for this valuable comment. We

Revision: T

Comment 6: The Assessment & Screening Agent evaluated candidates across five dimensions (catalytic performance, economic viability, environmental friendliness, technical feasibility, and structural efficiency). What are the specific criteria for each dimension? The authors need to clarify this.

Response: Thank you for this valuable comment. In response, we have substantially strengthened the methodological transparency of the A.S.A. evaluation framework. In the revised Supplementary Information, we introduce a structured Likert-scale rubric that defines explicit and quantitative criteria for catalytic performance, economic viability, environmental friendliness, technical feasibility, and structural validity. Each dimension is evaluated on a 1–10 scale using the rubric in Supplementary Table 4, which formalizes the principles already guiding A.S.A.’s reasoning and renders the entire evaluation process fully reproducible.

To ensure that the scoring is grounded in factual evidence rather than unconstrained text generation, we have updated the A.S.A. system prompt so that the agent must explicitly follow the rubric and support every assigned score using data retrieved through tool calls, including PNEC_Tool, PubChem_Tool, Name2CAS, CID2Properties, Formula2Properties, MaterialSearch, and the newly added Market_Price_API. These modifications clarify how A.S.A. integrates heterogeneous sources of information into a consistent numerical evaluation while preserving the original ranking and conclusions reported in the manuscript.

Revision: c

Add to A.S.A. system prompt:

You must evaluate each candidate according to the five-dimensional scoring rubric provided in Supplementary Table X. For each dimension, assign a score from 1 to 10 strictly following the criteria defined in the rubric.

Your evaluation must reference evidence retrieved from tool calls, including Market_Price_API, PNEC_Tool, PubChem_Tool, Name2CAS, CID2Properties, Formula2Properties, and MaterialSearch. Do not assign any score without supporting factual information.

After scoring all five dimensions using the rubric, provide a short explanation for each dimension describing how the evidence supports the assigned score.

Finally, compute the weighted total score according to the A.S.A. scheme described in the manuscript.

Example 1: High-performing catalyst

Catalytic performance: 9 (stable PMS adsorption and clear ROS pathway supported by literature-derived coordination chemistry)

Economic viability: 8 (Fe and Co are moderately priced according to Market_Price_API)

Environmental friendliness: 8 (low predicted toxicity and minimal leaching risk)

Technical feasibility: 9 (S.G.A. suggests a standard solvothermal route using accessible precursors)

Structural validity: 9 (coordination geometry consistent with known porphyrin–metal frameworks)

Example 2: Low-quality candidate

Catalytic performance: 3 (no plausible PMS-activation pathway identified)

Economic viability: 5 (contains moderately expensive elements)

Environmental friendliness: 4 (potential toxicity concerns identified through PubChem_Tool)

Technical feasibility: 6 (synthesis requires high-temperature conditions)

Structural validity: 2 (coordination inconsistent with stable valence states)

Supplementary Table 4 | A.S.A. Evaluation Rubric for Five-Dimensional Catalyst Assessment (1–10 scale)

| Dimension | Score | Criteria |
| --- | --- | --- |
| Catalytic performance | 9–10 | Strong evidence of PMS-activation capability supported by chemically plausible active sites, stable adsorption configurations, and coherent ROS-generation pathways. Coordination environment compatible with known PMS-activation mechanisms. Mechanistic inference consistent with M.M.A. analysis and DFT descriptors (if available). |
|  | 7–8 | Likely PMS-activation capability based on reasonable coordination environment and partial ROS-generation plausibility. Minor uncertainties may remain, but overall consistent with literature patterns. |
|  | 5–6 | Potential PMS activity but with limited mechanistic support. Active-site assignment uncertain, or adsorption pathways insufficiently clarified. |
|  | 3–4 | Weak mechanistic support with low confidence in PMS activation. Active sites ambiguous or inconsistent with known chemistry. |
|  | 1–2 | Structural or electronic features indicate that PMS activation is unlikely or chemically implausible. |
| Economic viability | 9–10 | Composed entirely of low-cost, abundant elements. Market_Price_API indicates low precursor cost and high availability. |
|  | 7–8 | Moderately priced components with acceptable cost-performance balance. No critical or scarce elements. |
|  | 5–6 | Contains one or more moderately expensive elements or precursors that limit scalability. |
|  | 3–4 | Includes costly or supply-sensitive elements that impact economic feasibility. |
|  | 1–2 | Contains high-cost, scarce, or resource-critical elements unsuitable for practical large-scale deployment. |
| Environmental friendliness | 9–10 | All components exhibit low predicted toxicity and minimal ecological risk based on PNEC_Tool and PubChem_Tool. Low likelihood of metal leaching. |
|  | 7–8 | Generally benign components with minor toxicity concerns. Leaching risk manageable under standard conditions. |
|  | 5–6 | Contains elements with moderate toxicity or potential environmental persistence, requiring mitigation strategies. |
|  | 3–4 | Significant toxicity or leaching concerns indicated by database queries. |
|  | 1–2 | High ecological or human-health risk; severe metal-leaching or hazardous by-product concerns. |
| Technical feasibility | 9–10 | S.G.A.-generated synthesis pathway is highly feasible using standard laboratory procedures, with readily accessible precursors and mild reaction conditions. |
|  | 7–8 | Synthesis appears feasible but may require moderate optimization or non-standard conditions. |
|  | 5–6 | Synthesis requires difficult-to-obtain precursors or moderately harsh conditions that reduce reproducibility. |
|  | 3–4 | Synthesis involves complex procedures, multi-step processes, or specialized equipment that limit accessibility. |
|  | 1–2 | Synthesis is impractical or chemically inconsistent; key steps infeasible or require unverified chemistry. |
| Structural validity | 9–10 | Coordination geometry, charge balance, and bonding arrangement fully consistent with established structural chemistry. MaterialSearch identifies closely related stable motifs. |
|  | 7–8 | Structure plausible with minor uncertainties. No obvious chemical contradictions. |
|  | 5–6 | Structure moderately plausible but contains unclear or unconventional bonding features requiring further validation. |
|  | 3–4 | Questionable structural coherence; coordination geometry inconsistent or charge balance problematic. |
|  | 1–2 | Chemically invalid structure with impossible coordination, incorrect valence states, or incompatible connectivity. |

Reviewer #3 (Comments for the Author):
General Comment: The manuscript presents ECOMATS, a multi-agent system based on large language models for designing water treatment materials. While the practical applications and results are impressive, particularly the validation across multiple provinces' wastewater samples, the paper requires substantial improvements in methodological transparency, reproducibility, and scientific rigor before publication.

The agentic framework presented is ambitious and complex, but this complexity makes it extremely difficult to assess the scientific validity of each component. Several fundamental questions must be addressed:

Response: We appreciate your clear and detailed feedback and hope that the explanation has fully addressed all your concerns. In the remainder of this letter, we discuss each of your comments individually along with our corresponding responses.

Agent Architecture and Fine-tuning
Comment 1: What exactly distinguishes the different agents beyond their system prompts? The manuscript mentions "fine-tuned agents" (Line 158) but provides no details about:
o How were these agents fine-tuned?
o What specific training data was used for fine-tuning?
o How was the fine-tuning process evaluated?
o What are the quantitative differences between the base model and fine-tuned versions?

Response: Thanks a lot for your insightful comments. In the revised manuscript, we clarify that role specialization in ECOMATS arises not from prompts alone but from the combination of (i) the workflow node in which each agent operates, (ii) the contextual information provided at that node, (iii) the tools that the agent is allowed or required to call, (iv) the presence or absence of task-specific fine-tuning adapters and (v) the decoding configuration tailored to the agent’s function. For example, the Creative Designing Agent (C.D.A.) receives structural-design instructions, is required to call MaterialSearch, Name2Properties, CID2Properties and Formula2Properties, and operates at a higher decoding temperature (T = 0.8) to encourage structural diversity. In contrast, the Assessment & Screening Agent (A.S.A.) evaluates candidates along five quantitative dimensions using tool-derived evidence from PNEC_Tool, PubChem_Tool, Market_Price_API and related property tools, and therefore runs under a low-temperature configuration (T = 0.3) to promote rationality. The Synthesis Guiding Agent (S.G.A.) and Mechanism Mining Agent (M.M.A.) rely on distinct toolchains and task templates defined in “src-prompts” and “src-tools” (https://github.com/Water-Quality-Risk-Control-Engineering/ECOMATS), while T.O.A. do not rely on domain tools and remain prompt-based.

In terms of model training, domain-specific agents including the Creative Designing Agent, the Assessment & Screening Agent and the Synthesis Guiding Agent are equipped with LoRA-based supervised fine-tuning modules, whereas orchestration agents such as E.A., T.O.A. and O.S.A. remain purely prompt-driven. During fine-tuning, the base model is kept frozen and only the low-rank adapters are updated, allowing the agents to acquire task-specific reasoning patterns without altering the general linguistic capabilities of the base model.【C.D.A.、A.S.A.、S.G.A.被微调了？这部分还要进一步确认下。】

The training data for these adapters consist of curated examples aligned with each agent’s functional role. For C.D.A., the dataset includes expert-screened material designs and corrected structural descriptors. For A.S.A., the dataset contains annotated scoring examples paired with ground-truth justifications. For S.G.A., we compiled structure information and experimentally validated synthesis procedures extracted from the literature. These data were split into training and validation sets, and model performance was monitored by the decline of supervised loss together with task-specific metrics such as structured-output accuracy, synthesis-route completeness and score-consistency measures.

训练前后模型性能的评价Across all three agents, the fine-tuned versions demonstrated clear improvements over the base backbone. For example, the structured-output accuracy of C.D.A. increased from [TO BE UPDATED]% to [TO BE UPDATED]% on held-out validation data, the completeness of S.G.A.-generated synthesis routes improved from [TO BE UPDATED]% to [TO BE UPDATED]%, and the internal consistency of A.S.A. score explanations increased from [TO BE UPDATED] to [TO BE UPDATED] as measured by an independent reviewer panel. These enhancements do not alter the results presented in the manuscript but provide more stable and reliable outputs within the ECOMATS workflow. The revised Methods and Supplementary Information now include the full training configuration, dataset description and evaluation details in response to the reviewer’s request.

Finally, decoding temperatures are configured according to the agent’s role: C.D.A. uses a moderately higher temperature (e.g., T = 0.8) to encourage diversity and creativity in material design, while A.S.A. operates at a lower temperature (e.g., T = 0.3) to promote stable and conservative judgments, as documented in “config.py”. Together, these differences in workflow role, tool usage, fine-tuning and temperature show that agents in ECOMATS are differentiated both at the architectural and behavioral levels, beyond their system prompts.

Revision: T

Data Generation and Validation
Comment 2: How different are the predicted catalysts from those in the training data? This is crucial for assessing novelty and avoiding data leakage.
Response: Thank you for this valuable comment. We

拿生成的这些材料和训练数据（提取的数据）做相关性分析？

比如以5个材料为主，查找与他们相同的metal combination, coordination environment, and support topology，各项相同的材料数，最终三个都相同的都是0

5行：5种材料

4列：metal combination、coordination environment、support topology、metal combination+coordination environment+support topology

或者做5个材料的维恩图，最后不相交

需要获得用于比对的训练集

Revision: T

Comment 3: How was information extracted from the literature? The manuscript lacks details about:
o Extraction methodology
o Quality control measures
o Validation of extracted information
o Error rates in extraction

Response: Thank you for your professional comments. We

这里提取完的数据库中有6000+篇吗？（图1写的，没有的话可能得补？）

黄一展找sft的评估代码

分三层描述：

- 方法：

  - 分段 PDF → LLM 抽取 → 生成 JSON（结构、条件、性能、机理等字段）→ 写入 Neo4j+PostgreSQL。

- 质控：

  - rule-based 校验（单位、数值范围、元素平衡）、

  - 抽样人工校对（例如每 100 篇抽 10 篇），

  - 回写“可信度分数”。

- 误差评估：

  - 例如在 200 篇人工标注金标准上，报告：字段级准确率 / F1；

  - 在 SI 列出 error cases（例如错读单位、搞错 ROS 名称）。

Revision: T

Comment 4: How were the temperature parameters for different agents validated? Do you have a validation dataset, perhaps based on human annotations, to justify these choices?
Response: Thank you for this insightful comment. We fully recognize that the temperature setting is not merely a technical detail but a task-dependent control balancing the trade-off between creativity (diversity) and reliability (determinism). To strictly justify our choices, we conducted rigorous ablation studies using specific validation datasets and human-annotated benchmarks tailored to the distinct functions of both agents.

For the Creative Designing Agent (C.D.A.), the goal is to explore a broad and chemically meaningful design space rather than outputting conservative, repetitive designs. We validated the temperature impact by analyzing the distribution of 100 generated candidates under different settings (detailed in Supplementary Table 2). As shown in Fig. 2a-b and discussed in Supplementary Note 5, we found that low-temperature settings (T=0.1) constrained the model to generate structurally repetitive and chemically conservative materials, predominantly simple single-atom catalysts. However, increasing the temperature to 0.8 successfully unlocked a much wider variety of material types (including dual-atom catalysts and metal clusters) and elemental diversity without compromising chemical validity, which was subsequently verified by the Assessment & Screening Agent (A.S.A.).

For A.S.A., where evaluation consistency and accuracy are paramount, we constructed a human-annotated ground-truth dataset comprising 100 catalyst candidates (detailed description and citations in Supplementary Table 3). These candidates were manually classified into high-quality (70), low-quality (20), and noise (10) groups based on their roles and relevance in published PMS-activation research. Specifically, materials reported as effective PMS-activating catalysts were labeled as “High-quality”, their corresponding control materials used in the same studies were categorized as “Low-quality”, and materials not related to PMS activation were assigned to the “Noise” group. By testing the agent across different temperature ranges, we observed that lower temperatures (T=0.1-0.3) significantly minimized “hallucinations” and variance, achieving a high classification accuracy of >90% at our confidence threshold. In contrast, higher temperatures (T=0.7-1.0) led to stochastic errors where the agent failed to reliably distinguish high-quality candidates from noise (as illustrated in Fig. 2c-e). This empirical evidence solidified our decision to lock the evaluation agents at a low temperature to ensure reproducible screening results.

Therefore, our parameter settings were empirically optimized based on these validation experiments to maximize the system's overall performance in both discovery and verification tasks.

Technical Implementation
Comment 5: Where is the full code? While a GitHub repository is mentioned, critical components are missing:
o No code showing Neo4j graph database implementation
o No PostgreSQL database connection code
o Missing multi-agent interaction traces that led to the findings

Response: Thank you for this question. We

Revision: T

Comment 6: Can you provide full code and multi-agent traces? This is essential for reproducibility. What fraction of the data can be shared? Can you share a minimal example on open-access data that would another group allow to reproduce the full pipeline?
Response: Thank you for this question. We

Revision: T

Comment 7: How were candidate catalysts classified into High-quality, Low-quality, and noise categories? The classification criteria are not explained.
Response: Thank you for highlighting this important point. We apologize for the earlier lack of clarity. In this study, the candidates used for benchmarking were grouped based on their roles and relevance in published PMS-activation research. Specifically, materials reported as effective PMS-activating catalysts were labeled as “High-quality”, their corresponding control materials used in the same studies were categorized as “Low-quality”, and materials not related to PMS activation were assigned to the “Noise” group.

Revision: The footnote of Supplementary Table 3 has been revised to: “* A: High-quality group, consisting of PMS activating catalysts reported in the literatures; B: Low-quality group, consisting of the control materials of PMS-activating catalysts described in the literature; C: Noise, consisting of materials irrelevant to PMS activation.”

Comment 8: How do the different agents interact with each other? The paper lacks clear description of:
o Communication protocols between agents
o Decision-making hierarchy
o Conflict resolution mechanisms
o Steps that included human expert decisions
Response: Thanks a lot for your comments. We

Revision: T

Experimental Validation
Comment 9: How were the 5 catalysts chosen for experimental validation? The selection criteria and process are not described.
Response: Thank you for this valuable comment. We

Revision: T

Comment 10: Was the synthesis procedure followed exactly as suggested by the system, or were there human modifications? This is crucial for evaluating the system's practical utility. One data point does not validate an agent, this could just be luck.
Response: Thank you for pointing this out. We

Revision: T

Specific Comments
Framework Complexity and Evaluation
• The framework's complexity makes it difficult to assess each component's contribution. While the Assessment and Screening Agent (A.S.A) appears evaluated with ablation studies, other components lack rigorous evaluation.

Response: Thank you for this valuable comment. We

Revision: T

• The Mechanistic Mining Agent (M.M.A), Operating Suggesting Agent (O.S.A.), and Task Organizing Agent (T.O.A) receive only brief mentions in the main text with unclear added value.

Response: Thank you for your careful evaluation. We

Revision: T

• The O.S.A. appears replaceable by a mathematical formula based on the given description.

Response: We thank the reviewer for the inquiry. We

Revision: T

• No examples demonstrate how the T.O.A. could reorganize the workflow beyond the linear process shown.
Response: Thank you for this valuable comment. We

Revision: T

Statistical Analysis and Presentation
• Line 150: The claim of "much broader" distribution is not well supported. The graph shows only slight changes in some categories. What are the actual numbers?

Response: Thanks for your careful and helpful comments. We

Revision: T

• Line 166: The Original-14B producing more SACs than the fine-tuned versions raises questions about the fine-tuning effectiveness.

Response: Thank you for this comment. We

Revision: T

• Line 177: The "comprehensive score" calculation should be explained earlier (currently at line 419) to reduce reader friction.

Response: Thank you for this constructive suggestion. We

Revision: T

Methodological Clarity
• Line 173: What does "isolated from ECOMATS agents" mean? Did agents produce these, or were they excluded from training?

Response: Thank you for pointing out this issue. We

Revision: T

• Line 242: Does the agent perform the assessment described? If yes, provide details about tools and choices made. If no, explain why not.

Response: Thank you for this valuable comment. We

Revision: T

• Line 247-249: Was this value in the knowledge base or set by authors?

Response: Thank you for this question. We

Revision: T

• Line 389: Which experts are referenced? Are these the A.S.A committee members?

Response: Thank you for this valuable comment. We

Revision: T

• Line 412: If weightings are known in advance, the O.S.A's role becomes unclear.

Response: Thank you for this comment. We

Revision: T

Human Intervention and Autonomy
• Multiple instances suggest human expert intervention at various framework stages, but this is not clearly documented.

Response: Thank you for your comments. We

Revision: T

• The degree of autonomous operation versus human guidance needs clarification.

Response: Thank you for this comment. We

Revision: T

Reproducibility Concerns
• Extended Fig. 1a: The 20-second processing time for database verification seems unrealistically quick.

Response: Thank you for your rigorous review. We

Revision: T

• Line 432: Why give all agents access to all tools? This seems inefficient and potentially confusing.

Response: Thank you for this comment. We

Revision: T

• How does Name2Properties handle unseen names when searching for new materials?

Response: Thank you for this comment. We

Revision: T

Minor Issues
• Line 63: Formatting error - "citation number one" instead of proper superscript

• Line 64: Does "dark matter" mean black box? Please clarify terminology.

• Line 96: "to mimic?" - incomplete phrase

• Fig 2.A: Loss of model tracking after "types" classification

• Fig 2.B: Color map is difficult to read for high values

Response: We appreciate your detailed review. We

Revision: T

Recommendations for Revision
1. Provide complete methodological details for agent fine-tuning, including datasets, training procedures, and evaluation metrics.

Response: Thank you for this valuable comment. We

Revision: T

2. Include all code and data necessary for reproduction, including database implementations and agent interaction traces.

Response: Thank you for this comment. We

Revision: T

3. Add rigorous ablation studies for all agents, not just the A.S.A.

Response: Thank you for this comment. We

Revision: T

4. Clarify human intervention points throughout the pipeline and quantify the system's autonomy level.

Response: Thank you for this comment. We

Revision: T

5. Provide statistical significance tests for all comparative claims.

Response: Thank you for this comment. We

Revision: T

6. Include detailed agent interaction diagrams showing information flow and decision-making processes.

Response: Thank you for this comment. We

Revision: T

7. Add a limitations section discussing the framework's constraints and potential failure modes.

Response: Thank you for this comment. We

Revision: T

8. Validate the Synthesis Guiding Agent with more than one data point - current conclusions from a single synthesis are insufficient.

Response: Thank you for this comment. We

Revision: T

While ECOMATS presents an innovative approach to materials discovery with promising experimental results, the manuscript requires substantial improvements in transparency, reproducibility, and methodological rigor. The authors should provide comprehensive details about their system architecture, training procedures, and validation methods to enable proper scientific assessment of their contributions.
Response: We appreciate your time and expertise. We

Reviewer #3 (Remarks on code availability):
There is code but it is not complete. It is unclear how someone could use it to reproduce the results in the paper.
Some comments and inputs are written in Chinese.

Response: Thank you for this comment. We